Raw Values = Calibration

Ultrasonic

| Parameter | Value |
| --- | --- |
| EMPTY_DISTANCE_CM | 59 cm |
| FULL_DISTANCE_CM | 4 cm |
| OFFSET (Correction) | 3.5 cm |
| Formula | (empty - distance) / (empty - full) * 100 |

Weight sensor 1

| Parameter | Value |
| --- | --- |
| RAW (Empty) | 451,977 |
| RAW (With 1.31 kg)** | 608,600 – 609,000 |
| RAW Difference | ≈ 156,600 |
| Known Weight | 1.31 kg |
| SCALE1 | 119,800 |
| Formula Used | (RAW_weight - RAW_empty) / weight |
| Verified Weight Reading | 1.308 – 1.326 kg |

Weight Sensor 2

| Parameter | Value |
| --- | --- |
| RAW (Empty) | -491,000 |
| RAW (With 1.31 kg)** | -336,700 |
| RAW Difference | ≈ 154,300 |
| Known Weight | 1.31 kg |
| SCALE2 | 117,786 |
| Formula Used | (RAW_weight - RAW_empty) / weight |
| Verified Weight Reading | 1.308 – 1.326 kg |

Gas Sensor

| Parameter | Value |
| --- | --- |
| RAW (Normal Air) | ~100–300 |
| RAW (Alcohol Spray) | ~600–900+ |
| Digital Trigger | Active LOW (pin 25) |